AGT-44 Seal Flush and Leak Inspection Notes

Mechanical maintenance inspection records for reactor agitator AGT-44. Values are transcribed from technician field notes and seal-panel readings.

## Seal flush and leakage records

| record_id | inspection_date | agitator_tag | gearbox_tag | motor_tag | component | measurement_type | item_description | unit | source_value | technician | record_note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEAK-001 | 2026-07-08 | AGT-44 | GBX-44 | MTR-44 | mechanical seal | seal leakage rate | seal leakage rate | ml/min | 6.2 | B. Adeyemi | graduated cylinder under gland drain |
| LEAK-002 | 2026-07-08 | AGT-44 | GBX-44 | MTR-44 | seal flush | seal flush flow | seal flush flow | L/min | 2.1 | B. Adeyemi | rotameter bouncing during batch heat-up |
| LEAK-003 | 2026-07-08 | AGT-44 | GBX-44 | MTR-44 | seal flush | seal flush pressure | seal flush pressure | bar | 1.3 | B. Adeyemi | panel gauge below normal zone |
| LEAK-004 | 2026-07-08 | AGT-44 | GBX-44 | MTR-44 | mechanical seal | mechanical seal kit part number | mechanical seal kit part number | text |  | B. Adeyemi | cartridge label unreadable from access side |
| LEAK-005 | 2026-07-09 | AGT-44 | GBX-44 | MTR-44 | seal pot | seal pot sight glass condition | seal pot sight glass condition | text | cloudy sight glass | M. Sato | visual condition only |
| LEAK-006 | 2026-07-09 | AGT-44 | GBX-44 | MTR-44 | seal flush | seal flush isolation tag | seal flush isolation tag | text | SFV-44A open | M. Sato | checklist mark |

## Lubrication and bearing observations

| record_id | inspection_date | agitator_tag | gearbox_tag | motor_tag | component | measurement_type | item_description | unit | source_value | technician | record_note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LUB-001 | 2026-07-03 | AGT-44 | GBX-44 | MTR-44 | gearbox | gearbox oil grade | gearbox oil grade | text | ISO VG 320 EP | M. Sato | oil drum label recorded before fill |
| LUB-002 | 2026-07-03 | AGT-44 | GBX-44 | MTR-44 | gearbox | gearbox oil fill quantity | gearbox oil fill quantity | L |  | M. Sato | litre count not written on lubrication note |
| LUB-003 | 2026-07-09 | AGT-44 | GBX-44 | MTR-44 | gearbox breather | breather condition | breather condition | text | dust loaded | T. Banner | visual observation |
| LUB-004 | 2026-07-09 | AGT-44 | GBX-44 | MTR-44 | gearbox sight glass | sight glass condition | sight glass condition | text | foaming visible | T. Banner | visual observation |
| LUB-005 | 2026-07-09 | AGT-44 | GBX-44 | MTR-44 | motor DE bearing | motor DE bearing part number | motor DE bearing part number | text |  | T. Banner | end cover not opened |
| LUB-006 | 2026-07-09 | AGT-44 | GBX-44 | MTR-44 | gearbox DE bearing | bearing temperature | gearbox DE bearing temperature | degC | 84.0 | T. Banner | contact probe |

## Descriptive inspection notes

| note_id | inspection_date | agitator_tag | component | note_category | note_text | record_handling |
| --- | --- | --- | --- | --- | --- | --- |
| N-001 | 2026-07-08 | AGT-44 | mechanical seal | observation | fine wet band under lower gland drain | text-only unless timed leak value exists |
| N-002 | 2026-07-09 | AGT-44 | coupling guard | observation | guard lower lip has beige powder deposit | text-only observation |
| N-003 | 2026-07-09 | AGT-44 | gearbox | observation | oil smell stronger than prior week | text-only observation |
| N-004 | 2026-07-10 | AGT-44 | seal flush | observation | seal pot sight glass cloudy after one hour circulation | text-only observation |